# Required Components

| 1 | 2 3 | BILL OF MATERIALS 1 1x Cool Beats Penguin 2 2x Mono jack 3 4x 10 cm pieces of wire |
| --- | --- | --- |

# Required Tools

- Drill with ¼” drill bit

- Wire Strippers

- Flush Cutters

- Phillips screwdriver

- Soldering Iron

- Solder

- Ruler

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

| Carefully take the penguin out of the package. The toy will be put back in the packaging once adapted. 01. | Remove and set aside the 6 screws from the base of the penguin. 02. |
| --- | --- |
| Set aside the red and green buttons. 03. | Cut 4 lengths of wire about 10cm long. 04. |
| X4 Strip 1 cm of the insulation off each end of the wires. 05. | Solder a wire to the first leg of a mono jack, solder a second wire to the second leg of the mono jack. Repeat with the second mono jack and other wires. 06. |
| Open the base of the penguin. Locate the left circuit board. 07. | A Solder the other ends of the wires from one mono jack to the locations shown. 08. |
| Locate the right circuit board. 09. | B Solder the other ends of the wires from the second mono jack to the locations shown. 10. . |
| With a ¼” drill bit, drill a hole in the back end of the penguin’s foot. Make sure there is enough room for a mono jack to fit inside. 11. | Drill another hole on the other side. Next, remove the nut from mono jacks and push the mono jacks through the holes. Reinstall the nuts and tighten to secure. 12. |
| Reinsert the green and red buttons. 13. | Reinsert the 6 screws into the bottom of the penguin. 14. |
| Once the toy is tested with a switch, it can be repackaged. 15. |  |